DA40  DIAMOND STAR Normal Procedures

(incl high altitude)

### PRE-START

Documents/Preflight Inspection– CHECK

Weight/Balance/Dens. Alt – CHECK

Hobbs Time – Logged

Baggage – STOWED

Rudder Pedals – ADJUSTED/LOCKED

Passenger Briefing – PERFORMED

Rear Door – CLOSED and LOCKED

Harness – FASTENED AND ADJUSTED

Parking Brake – SET

Flight Controls – FREE & CORRECT

Fuel Valve – EMPTIEST TANK

Trim Wheel – T/O

Mixture – IDLE CUT-OFF

Prop Control – FULL FORWARD

Throttle -  FREE & IDLE POSITION

Cabin Heat – OFF

Circuit Breakers – CHECK IN

Alternate Air  - CLOSED

Alternate Static Valve - CLOSED

Avionics Master – OFF

Ess. Bus Switch - OFF

Electrics – OFF

ALT/BAT Master Switch – ON

Canopy – CLOSED and SECURE

Strobes – ON

## STARTING ENGINE

# Cold Start

Throttle – SET 1 Inch

Propeller – FULL FORWARD

Mixture – IDLE CUT-OFF

Brakes – HOLD

Propeller Area – CLEAR

Fuel Pump - ON

Mixture – FULL RICH                       (3 - 5 Sec.) IDLE CUT-OFF

Fuel Pump - OFF

Throttle – SET TO ½  INCH

Ignition – START POSITION

Mixture – ADVANCE

(engage starter 10 sec MAX

and allow 20 sec. to cool)

# Warm Start

Throttle – SET 1 Inch

Propeller – FULL FORWARD

Mixture – IDLE CUT-OFF

Brakes – HOLD

Propeller Area – CLEAR

Fuel Pump - ON

Mixture − IDLE CUT-OFF

Throttle – ½  inch OPEN

Ignition – START POSITION

Mixture  –  ADVANCE

(engage starter 10 sec MAX

and allow 20 Sec to cool)

# Upon Engine Start

Throttle – min 1000 PRM

Fuel Pump – OFF

Oil Pressure – CHECK GREEN

Ammeter – CHECK GREEN

Fuel Pressure – MIN 14 PSI

Oil Pressure – GREEN IN 15 SEC.

If not shutdown

Annunciator Panel  – CHECK CLEAR

Engine Gauges – GREEN

Mixture – LEAN 1”+

Fuel Pump – OFF

Fuel Computer -- UPDATE

## BEFORE TAXIING

Avionics Master Switch – ON

Aircraft Lighting – AS REQUIRED

ATIS – COPY

Flight Instruments  - SET

HSI – TO COMPASS

Radios/GPS - SET

Transponder – STANDBY /1200

Engine Gauges – CHECK GREEN

Annunciator Panel – CHECK CLEAR

Autopilot – ON, OVERPOWER

Autopilot – ON, Stick Button Disc.

Fuel Valve – SWITCH TANKS

Taxi Clearance -- OBTAIN

Parking Brake – RELEASE

## TAXIING

Brakes – Check

Flight Instruments – TAXI CHECK

# RUN UP

Line Up Into Wind

Parking Brake – SET

Nose Wheel – STRAIGHT

Brakes – HOLD

Oil Temp. – CHECK GREEN

Oil Pressure – CHECK GREEN

Propeller – FULL FORWARD

Throttle – 2000 RPM

Mixture – MAX POWER GUESS

Check Ignition

LEFT/BOTH/RIGHT/BOTH

MAX DROP 175 RPM

MAX DIFF 50 RPM

Propeller – CYCLE 3 TIMES

Max Drop 500 RPM

Alternator Load – CHECK (1/day)

Mixture – MAX POWER + 1”

Engine Gauges – CHECK GREEN

Throttle – IDLE TEST

Throttle – 1000 RPM

Alternate Air – TEST

Annunciator Panel – CHECK CLEAR

# BEFORE TAKE – OFF

Safety Belts – FASTENED

Canopies – F LOCK, R LOCK, LIGHT

Fuel Valve – CHECK

Fuel Quantity – CHECK

Mixture – MAX POWER+

Propeller – FULL FORWARD

ALT/BAT Master Switch – ON

Ignition Switch – BOTH

Engine Instruments – CHECK

Flight Instruments – SET

Flaps – VERIFY T/O POSITION

Trim – SET FOR T/O

Autopilot – OFF

Annunciator Panel – CHECK CLEAR

Flight Controls – FREE/CORRECT

Fuel Pump – ON

Landing Light - ON

Transponder – 1200/Standby

Strobes (Canada) – ON

Parking Break – RELEASE

# TAKE-OFF

Throttle – FULL POWER

(min 2660 RPM)

Engine Temp/Pressure - CHECK

Rotate – 59 kts

Climb Speed – 66 Kts (60 solo)

AFTER TAKE-OFF (500’ AGL)

Prop – 2400 RPM

Flaps – UP

Climb Speed -- 73 Kts (68 solo)

1000’ AGL

Fuel Pump – OFF

Landing Light – OFF

### CRUISE

### Throttle – SET AS DESIRED (22”)

Propeller – 1800-2400 RPM

Mixture – LEAN (as required)

Fuel Valve – FULLEST TANK

Annunciator Panel – CHECK CLEAR

### BEFORE LANDING

GEN/BAT Master Switch – ON

Ignition –BOTH

Gauges – GREEN

Breakers –CHECK IN

Landing light – ON

GUMPS

Fuel Valve – FULLEST TANK

Fuel Pump – ON

Mixture – RICH

Prop – 2400 RPM

Seatbelts – FASTENED

### FINAL APPROACH

Flaps – LANDING

Airspeed – 67  (63 solo)

Mixture – RICH

Propeller – FULL FORWARD

### GO-AROUND

Throttle – FULL

Flaps – T/O

Airspeed – 66 (60 solo)

### CLEAR OF RUNWAY

Throttle – 1000 RPM

Flaps – UP

Strobes – OFF (Canada)

Landing Light – OFF

Fuel Pump – OFF

Mixture – Lean 1”+